COURSE MEASUREMENT REPORT

- REPORT FOR NATIONAL CERTIFIED COURSE –

GENERAL INFORMATION

Event : Kalandseid Løpskarusell

Arrangør: Kalandseid IL

Location : Bergen

Date of event : June 2, 2025

Date and time for measurement: 22.05.2025, 11.00 AM

Distance to be measured : 3K, 5K and 10K

Length of calibration course : 350 m

Terrain: Flat Slightly hilly Hilly Terrain Type of course route: Closed loop Back and forth A to B Other Race surface: Asphalt Elevation Start - Finish: 0m Description of start: : Lamp post at start of bicycle path ( Figure 3 ) Description of finish: Same as Start Distance, in a straight line, between start & finish: 0m Start and Finish marked: Yes No Km-markings: Yes No Measurement direction: Running direction Opposite running direction National record can be approved: Yes No

| CONTACT INFORMATION |  |  |
| --- | --- | --- |
|  | Measurer | Organizer: KALANDSEID IL |
| Kontaktperson: | Ivar Andreas Sandø | Erik Tonheim |
| Telefon: | +47 91648793 | +47 96626546 |
| Epost: | Ivarsando53@gmail.com | erik@kalandseid.no |

Det bekreftes herved, at løypa er oppmålt av en nasjonal sertifisert løypemåler fra Norsk friidrett. Løypa er oppmålt i henhold til Norsk friidretts bestemmelser, og distansen(e) har de(n) korrekte lengde(r). Oppmålingen gjelder i 5 år med unntak hvis traseen endres.

23.05.2025 Ivar A. Sandø

Dato	   	 Løypemålers underskrift			Dato	      	NFIFs godkjennelse

| CALIBRATION COURSE INFORMATION ( Establishe in 2024 by Ivar Sandoe and Geir Jensen) |  |
| --- | --- |
| Length of measuring tape (steel measuring tape only) | 50m |
| Length of calibration course 1st measurement | 350,00m |
| Diifference between 1st and 2nd measurement | 0,02m |
| Possible 3. measurement | N/A |
| Cooose midpoint as 350.00 m mark | 350,00 m |
| Temperature (road or measuring tape) | +20C |
| Adjustment, jfr. D-(D*(1+(temp-20)*0,0000115) | 0 cm |
| Measured calibration distance after adjustment | 350,00 m |
| Distance from start/finish to calibration distance | 10k |

| CALIBRATION INFORMATION - (max 8 counts difference of 1000 m between 4 measurements, ie 2 counts of 300m and 3 count of 400m) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pre measurement calibration |  |  | Post measuremnet calibration |  |  |
|  | Date and time: 22/5 10.45 AM |  |  | Date and time: 22/5 12.15 PM |  |  |
|  | Start | Stop | Diff. | Start | Stop | Diff. |
| 1. Measurem. | 73363 | 81264 | 3901 | 35922 | 39825 | 3903 |
| 2. Measurem. | 81264 | 85165 | 3901 | 39825 | 43726 | 3901 |
| 3. Measurem. | 85165 | 89066 | 3901 | 43726 | 47628 | 3902 |
| 4. Measurem. | 89066 | 92968 | 3902 | 47628 | 51530 | 3902 |

Pre measurem.   Post mesurem.

| Average (1+2+3+4) / 4 | 3901,25 | 3902 | Today's constant. Either highest or average of before and after measurement |
| --- | --- | --- | --- |
| Counts 1 km | 11146,4 | 11148,6 |  |
| Addition of 1‰ | 11,14 | 11,15 |  |
| Constant (rounded o integer) | 11158 | 11160 | 11159 |

| MÅLEPROSEDYRE |  |  |
| --- | --- | --- |
| Measuring constant 11158 |  |  |
| Measured distance (km) | Counter | Description of podition |
| 0 | 98817 | Start |
| 1,5 k | 115554 | Turning point 3K ,Figure 4 |
| 2,5K | 126712 | Turning point 5K, Figure 5 |
| 5K | 93115 | Turning point 10K, Figure 6. |

Measuremnet description

- Measurement method Measured using a Cook-Jones counter mounted on front whee The course has identical start and finish line. ( Figure 3) The course was measured in running direction from start line to the turning points for 3K, 5K and 10K. Deviations from shortest path : NONE - Determination of course length The final constant is 11159 counts/km. The measuring constant was 11158. This represents 9 cm / km. The turning points established in the measurements are adjusted accordingly ( away from start/finish line)

Figure 1 : Calibration course

Figur 1 :350m calibration course in Straumevegen

The calibration length is 350,00 m and marked with permanent spray og spikes on asphalt

Figure 2 : Course Map

The course is a back and forth course, following the bicycle/footpath from Kalandseid to Hamre and back. The whole width of the path can be used in both directions.

Figur 3 :     Start and finish line at Kalandseid

Start and finish line at Kalandseid – At the lamp post at start of the bicycle path

Figur 4 :  Turning point 3K

Turning point at 1.5 K : 0.4m ahead of the permanent marker post.

Figur 5 :  Turning point 5K

Turning point 5K  : 0.8m ahead of the permanent marker post.

Figur 6 : Turning point 10K

Turning point 10K :   0,5m after the permanent marker post